SYNCHRONOUS FIFO — VERIFICATION PLAN

Author: Trần Hồ Khánh Tân

DUT: sync_fifo.v  |  Reference: FIFO_Design_Spec.docx

| Number | Name block | Feature | Verification item | Test Method | Pass condition |
| --- | --- | --- | --- | --- | --- |
| 1 | Reset | Async reset | - rst_n, rd_ptr, wr_ptr, r_count, af_flag, ae_flag | - Put rst_n go low ( 0 ) while during a random operation ( write, read ) and has random data. After that, rst_n go back high (1). | - The signal : rd_ptr, wr_ptr, r_count immediately went low ( 0) . Then after release, full = 0, empty = 1, af_flag = 0, ae_flag = 0. |
| 2 | Write | Single write | - wr_en, wr_ptr, wdata, r_count, empty | - After reset, assert wr_en = 1 and apply a valid wdata value while full = 0. Observe the FIFO at the rising edge of clk. | - The write transaction is accepted at the rising edge of clk when wr_en = 1 and full = 0. - wdata is stored at the current wr_ptr, and wr_ptr increments by one. - r_count increments from 0 to 1 after the valid write. - empty deasserts (changes from 1 to 0) in the following clock cycle, indicating that the FIFO contains one valid data element. |
| 3 | Write | Write until full from empty | - wr_en, wr_ptr, wdata, full, af_flag, ae_flag | - Write consecutive values ( L value) when wr_en = 1 and full = 0 ( wr_en && !full) | - All values readable in order - The flag full go high after (L + 1) - th writes cycles. - af_flag go high after ( L - 2) - th write cycles. - ae_flag first go high until the second write cycles then go low at the next rising edge of clock. |
| 4 | Write | Write when full | - wr_en , wr_ptr, wdata, full, af_flag, r_count | - Write 1 value ( wdata) when wr_en = 1 and full = 1 ( wr_en && full) | - r_count unchanged - The write operation be ignored - full flag unchanged still in high - af_flag unchanged still in high - wr_ptr unchanged. - memory unchanged. |
| 5 | Write | Pointer wrap around(write) | - wr_ptr | - Write more than L - th values with read interleaved | - wr_ptr will point back to 0 after L-1 |
| 6 | Write | Write to check af_flag | -af_flag | - Give wr_en && !full. Write from L-2 to L - th . After that, read all the values out of the memory | - af_flag will go high at the begin of the (L - 2) -th and keep it high untill L -th . After the read, from the ( L - 3) the af_flag will go low ( 0) |
| 7 | Read | Single read | Rd_en, rd_ptr, rdata,full | - Write 1 value ( wdata) when memory empty and after that read back the value | - The read transaction is accepted at the rising edge of clk when rd_en = 1 and empty = 0. - rd_ptr increase by 1 - Check the rdata if it match the wdata that have been written in - empty deasserts in the write cycle (changes from 1 to 0) in the following clock cycle . After that , in the read cycles, empty change from 1 to 0 in the following clock cycle - r_count decrements from 1 to 0 after the valid read. |
| 8 | Read | Read until empty from full | empty,ae_flag, rd_ptr, rdata,full | - Write the value ( wdata) from empty to full the FIFO. After the , start the read cycles ( rd_en = 1 and empty = 0) until the FIFO empty. | - Check the rdata if it match the values(wdata) that have been written in. - ae_flag go high when there are 2 values left in FIFO and keep it high until it empty. - empty go high in the following clock cycle after the last value be read - full change from ( 1 to 0) in following clock cycle after the rd_en go high one cycle |
| 9 | Read | Read when the FIFO empty | Rdata,r_count,rd_ptr | - Read the FIFO when rd_en = 1 and empty = 1. | - r_count unchanged - rdata keep invalid not accept the value - rd_ptr unchanged |
| 10 | Read | Pointer wrap around ( read) | Rd_ptr | Read more than L -th values with writes in between | - rd_ptr correctly wraps to 0 after reaching L-1 |
| 11 | Counter | Simultaneous R/W | R_count, empty,full | - Asserts wr_en and rd_en with the satisfied condition together for several cycles | - r_count unchanged during this overlap - full and empty do not toggle during operation |
| 12 | Counter | Counter accuracy | R_count | - Give random valid sequence of write and read operation ( for example : 5 write, 3 read) | - Check the r_count match the all operation ( r_count = 2) |
| 13 |  |  |  |  |  |
| 14 |  |  |  |  |  |
| 15 |  |  |  |  |  |
| 16 |  |  |  |  |  |
| 17 |  |  |  |  |  |
| 18 |  |  |  |  |  |
| 19 |  |  |  |  |  |
| 20 |  |  |  |  |  |

Table: Verification test case list — fill in Number, Name block, Feature, Verification item, Test Method, and Pass condition per test case.

Notes / Column guide

- Number: sequential test case number (1, 2, 3, ...).

- Name block: which functional block this test targets (e.g. Reset, Write, Read, Counter, Flag).

- Feature: the specific feature or signal behavior under test (e.g. af_flag threshold, pointer wrap-around, write when full).

- Verification item: what exactly is being exercised — the specific condition, register, or signal state (e.g. r_count == af_level, wr_ptr wraps from L-1 to 0).

- Test Method: sequence of stimulus applied to the DUT (e.g. write random values continuously until full, assert wr_en while full=1, then read back).

- Pass condition: the expected output/state that confirms correct behavior (e.g. full asserted one cycle after r_count reaches L; rdata unchanged when reading during empty).